AT Command Summary

The AT Command Format

AT Commands are issued to the modem in the following format:

AT {command}{argument} [CR]

where:

AT  are the first letters of command letting the modem know that the succeeding string are commands

{command}  is an alphabet letter indicating the desired command

{argument}  is an optional number, letter or string of characters that further defines the command

[CR]  is the carriage return (also called the ENTER key or RETURN key)

Additional commands may be placed in a single command string:

AT {command}{argument} *** {command}{argument}  [CR]

The number of characters in a command string may not exceed 80.  If no argument is provided with a command that takes a numerical argument, an argument of zero is assumed

AT Command Set

Command	Function

AT	Attention - this precedes all commands except A/

A/	Execute previous command - does not require a <CR> *

A	Causes the modem to go off hook. If a call is coming in, the modem will try to answer it. The 	procedure for answering a call is a short silence and then an answer tone. Sending a character to 	the modem during this procedure will abort the answer procedure. The amount of time the modem 	will wait for a carrier is programmable by modifying the S7 register.

B0	Select CCITT V.22 (1200 bps)

B1	Select Bell 212A (1200 bps)

B2	Select CCITT V23

Originate mode will transmit data at 75 bps and receive data at 1200 bps.

Answer mode will transmit data at 1200bps and receive data at 75bps. The command N0 (Disable 	auto mode) must be selected.

D	D alone will take the modem off-hook and wait for a dial tone. (See X command for exceptions) 	The length of time to wait for a dialtone before dialing is programmable in register S6.

Dmn	ATDmn will dial a phone number where m is a modifier: L, W, ,, ;, @, !, or S. It will dial the 	telephone number n.

L	Dial last number

W	Wait for dial tone. If you have selected X0 or X1 (disable dial tone detection), then you can use 	this modifier to override that setting.

,	Pause during dial. The amount of time to pause is determined in register S8.

;	Return to command mode after dialing. It doesn't wait for carrier or hang up.

@	Wait for 5 seconds of silence. This is used to access systems that do not provide a dial tone.

!	Hook flash. Causes the modem to go on-hook for 0.5 seconds. This is used in PBX systems and for 	voice features like call waiting.

S=(0-9)	Dials a stored number. Up to ten numbers can be stored, and the addresses are from 0 to 9. To store 	a number into one of these addresses, use the &Z command.

^	Turns on 1300Hz calling tone.

E0	Commands issued to the modem are not echoed to the local terminal. This only matters in the 	command mode. It does not affect the modem's ability to send response codes.

E1	Commands are echoed to the local terminal.

AT Command Set continued

H0	Force modem on-hook (hang-up).

H1	Force modem off-hook (to answer or dial).

I0	Return numeric product code

I1	Return hardware variation code

I2	Report internal code

I3	Report software revision number

I4	Report product feature listing

L0	Speaker volume zero

L1	Speaker volume low

L2	Speaker volume low

L3	Speaker volume low                           (Hardware currently limits volume adjustment to on/off)

M0	Speaker always off

M1	Speaker on until carrier detected

M2	Speaker always on

M3	Speaker on during answering only

N0	Disable auto-mode. This forces the modem to connect at the speed specified in register S37.

N1	Enable auto-mode. The modem will answer at the highest available line speed and ignore 	any ATBn command.

O0	Return to data mode. If you have entered the command mode using the time independent escape 		sequence, this will put you back in data mode without going on-hook.

O1	Retrain the modem.  If the line condition has changed since the original connection, retraining the 	modem will cause it to reconnect at the most efficient speed for the current line condition.

P	Pulse dialing allows the modem to work on telephone networks where tone is not supported.

Q0	Enable response to DTE.

Q1	Disable response to DTE. The modem does not respond to the terminal. Issuing a command will 	not produce a response (unless the command is something like ATZ, which will restore this setting 	to default.)

Sn	Set default S-register. Any subsequent = or ? commands will modify the default S register.

Sn=m	Set register n to value m

Sn?	Return the value of register n

T	Tone dialing

V0	Result codes will be sent in numeric form. (See the result code table)

V1	Result codes will be sent in word form. (See the result code table.

W0	Report DTE speed only.  After connection, there will be  no message about what Error 	Correction or Data Compression protocol is in use.

W1	Report DCE speed, Error Correction/Data Compression protocol, and DTE speed.

W2	Report DCE speed only

X0	Send OK, CONNECT, RING, NO CARRIER, ERROR and NO ANSWER. Busy and Dial Tone 	Detection are disabled.

X1	Send X0 messages and CONNECT speed

X2	Send X1 message and NO DIALTONE

X3	Send X2 messages except NO DIALTONE, BUSY and RING BACK

X4	Send all responses

Y0	Disable long space disconnect

Y1	Enable long space disconnect; with error correction, hang up after sending 1.6 second long space; 	without error correction, hang up after 4 second long space.

Z0	Reset modem to profile 0

Z1	Reset modem to profile 1

=n	Sets the value of the default S register

?	Reports the value stored in the default S register.

AT Command Set continued

& 	Ampersand Commands

&C0	Force DCD on

&C1	DCD follows remote carrier

&D0	DTR is assumed on

&D1	DTR drop causes modem back to command mode without disconnecting

&D2	DTR drop causes modem to hang up

&D3	DTR drop causes modem to be initialized;&Y determines which profile is loaded.

&F	Load factory profile

&G0	Disable guard tone

&G1	Enable 550Hz guard tone

&G2	Enable 1800Hz guard tone on answering modem

&K0	Disable flow control

&K3	Enable RTS/CTS flow control

&K4	Enable XON/XOFF flow control

&K5	Enable transparent software flow control

&K6	Enable both RTS/CTS and XON/XOFF flow control

&P0	Make/break pulse dial ratio of 39/61

&P1	Make/break pulse dial ratio of 33/67

&S0	Force DSR on

&S1	DSR on at the start of handshaking and off after carrier loss

&T0	Terminate test

&T1	Start ALB test

&U0	Enable trellis coding

&U1	Disable trellis coding

&V0	Display active profile

&V1	Display stored profiles

&V2	Display stored telephone numbers

&W0	Save active profile to profile 0

&W1	Save active profile to profile 1

&Y0	Use profile 0 on powerup

&Y1	Use profile 1 on powerup

&Zn=m	Save telephone number (up to 36 digits) into memory location n (0-9)

AT Command Set continued

%	Percent Commands

%C0	Disable data compression

%C1	Enable MNP5 compression

%C2	Enable V.42bis compression

%C3	Enable both V.42bis and MNP5

%E0	Disable auto-retrain

%E1	Enable auto-retrain

%E2	Enable auto-retrain and fallback

%E3	Enable auto-retrain and fast hang up

%L	Report received signal level in

-dBm

%N0

%N1

%N2

%N3

%N4

%N5

%N6

%N7

%N8

%N9	Dynamic CPU loading disabled

Dynamic CPU loading not to exceed 10%

Dynamic CPU loading not to exceed 20%

Dynamic CPU loading not to exceed 30%

Dynamic CPU loading not to exceed 40%

Dynamic CPU loading not to exceed 50%

Dynamic CPU loading not to exceed 60%

Dynamic CPU loading not to exceed 70%

Dynamic CPU loading not to exceed 80%

Dynamic CPU loading not to exceed 90%

%Q	Report line signal quality

\	Backslash Commands

\A0	64-character max. MNP block size

\A1	128-character max. MNP block size

\A2	192-character max. MNP block size

\A3	256-character max. MNP block size

\Bn	In non-error correction mode, transmit break in 100 ms units (1-9 with default 3)

\G0	Disable XON/XOFF flow control (modem to modem)

\G1	Enable XON/XOFF flow control (modem to modem)

\Kn	Define break type (refer to table 2)

\L0	Use stream mode for MNP

\L1	Use interactive block mode for MNP

\N0	Normal mode; speed control without error correction

\N1	Plain mode; no speed control and no error correction

\N2	Reliable mode

\N3	Auto-reliable mode

\N4	LAPM error correction only

\N5	MNP error correction only

AT Command Set continued

AT\Kn Command

\Kn	Local DTE sends break during normal or reliable mode	Local modem sends break during 	plain mode	Remote modem sends break during normal mode

\K0	Enter command state; no break to remote	Break to remote; and enter command state	Empty 	data buffers; and send break to DTE

\K1	Empty data buffers; break to remote	Same as\K0	Same as\K0

\K2	Same as\K0	Send break to remote	Immediately send break to DTE

\K3	Immediately send break to remote	Same as\K0	Same as\K2

\K4	Same as\K0	Same as\K2	Send break to DTE with buffered RXD data

\K5	Send break to remote with TXD data	Same as\K2	Same as\K4

Result codes

Long Form	Short Form	Description

OK		0	Modem successfully executed a AT command

CONNECT	1	A connection established

RING		2	Modem detected an incoming call

NO CARRIER	3	Modem lost or could not detect a remote carrier signal within the register S7 			time

ERROR			4	Modem detected an error in an AT command

CONNECT 1200	5	Connection at 1200 bps

NO DIALTONE	6	Modem did not detect a dial tone within 5 seconds after off-hook

BUSY		7	Modem detected a busy tone

NO ANSWER	8	Modem did not detect 5 seconds of silence when using the @ dial modifier in 			the dial command

CONNECT 0600	9	Connection at 600 bps

CONNECT 2400	10	Connection at 2400 bps

CONNECT 4800	11	Connection at 4800 bps

CONNECT 9600	12	Connection at 9600 bps

CONNECT 7200	13	Connection at 7200 bps

CONNECT 12000	14	Connection at 12000 bps

CONNECT 14400	15	Connection at 14400 bps

CONNECT 19200	16	Connection at 19200 bps

CONNECT 38400	17	Connection at 38400 bps

CONNECT 57600	18	Connection at 57600 bps

CONNECT 115200 19	Connection at 115200 bps

CONNECT 28800	20	Connection at 28800 bps

CONNECT 300	21	Connection at 300 bps

CONNECT 1200TX/75RX	22	Connection at transmit 1200/receive 75 bps

CONNECT 75TX/1200RX	23	Connection at transmit 75/receive 1200 bps

CONNECT 110	24	Connection at 110 bps

RING BACK	25	Ring Back signal detected

+FCERROR	+F4	Error occured in Class 1 fax operation

FAX		33	Fax modem connection established

DATA		35	Data modem connection established

CARRIER 300	40	Carrier rate of 300 bps

CARRIER 1200/75 44	Carrier rate of transmit 1200/receive 75 bps

CARRIER 75/1200 45	Carrier rate of transmit 75/receive 1200 bps

CARRIER 1200	46	Carrier rate of 1200 bps

CARRIER 2400	47	Carrier rate of 2400 bps

CARRIER 4800	48	Carrier rate of 4800 bps

CARRIER 7200	49	Carrier rate of 7200 bps

CARRIER 9600	50	Carrier rate of 9600 bps

CARRIER 12000	51	Carrier rate of 12000 bps

CARRIER 14400	52	Carrier rate of 14400 bps

CARRIER 16800	53	Carrier rate of 16800 bps

CARRIER 19200	54	Carrier rate of 19200 bps

CARRIER 21600	55	Carrier rate of 21600 bps

CARRIER 24000	56	Carrier rate of 24000 bps

CARRIER 26400	57	Carrier rate of 26400 bps

CARRIER 28800	58	Carrier rate of 28800 bps

CONNECT 16800	59	Connection at 16800 bps

CONNECT 21600	61	Connection at 21600 bps

CONNECT 24000	62	Connection at 24000 bps

CONNECT 26400	63	Connection at 26400 bps

COMPRESSION: CLASS 5	66	MNP Class 5 data compression connection established

COMPRESSION: V.42bis	67	V.42bis data compression connection established

COMPRESSION: NONE	69	Connection established without data compression

PROTOCOL: NONE	76	Connection established without error correction

PROTOCOL: LAPM	77	V.42/LAPM error correction connection established

PROTOCOL: ALT	80	MNP 3-4 error correction connection established

S Registers

REG.	 Function

0	Rings to auto-answer

Set the number of rings required before the modem answers. 0 setting disables auto-answer.

Range: 0 - 255 rings

Default: 0 (auto-answer disabled)

1	Ring counter

Count the number of rings before the modem answers.

Range: 0 - 255 rings

Default: 0

2	Escape character

Define the character used for the three-character escape code sequence. 0 setting disables the escape code character.

Range: 0 - 127

Default: 43 (+)

3	Carriage return character

Define the character for carriage return

Range: 0 - 127

Default: 13 (carriage return)

4	Line feed character

Define the character for line feed

Range: 0 - 127

Default: 10 (line feed)

5	Backspace character

Define the character for backspace

Range: 0 - 127

Default: 8 (backspace)

6	Wait before dialing

Set the length of time to pause after off hook before dial.

Range: 2 - 255 seconds

Default: 2 seconds

7	Wait for carrier after dial

Set the length of time that the modem waits for a carrier from the remote modem before hanging up.

Range: 1 - 255 seconds

Default: 50 seconds

8	Pause time for dial delay

Set the length of time to pause for the pause dial modifier ",".

Range: 0 - 255 seconds

Default: 2 seconds

9	Carrier detect response time

Define the length of time a signal is detected and qualified as a carrier.

Range: 1 - 255 tenths of a second

Default: 6 (0.6 second)

10	Lost carrier hang up delay

Set the length of time the modem waits before hanging up for a carrier loss.

Range: 1 - 255 tenths of a second

Default: 14 (6 seconds)

11	DTMF speed control

Set the length of tone and the time between tones for the tone dialing.

Range: 50 - 255 milliseconds

Default: 95 milliseconds

12	Escape Prompt Delay (EPD) timer

Set the time from detection of the last character of the three character escape sequence until the "OK" is returned to the DTE

Range: 0 - 255 fiftieths of a second

Default: 50 (1 second)

13	Reserved

14	Reserved

15	Reserved

16	Reserved

17	Reserved

18	Test timer

Set the length of loopback test.

Range: 0 - 255 seconds

Default: 0 (disable timer)

19	Reserved

20	Reserved

21	Reserved

22	Reserved

23	Reserved

24	Reserved

25	Delay to DTR

Set the length of time the modem ignores DTR before hanging up

Range: 0 - 255 hundredths of a second

Default: 5 (0.05 second)

26	Reserved

27	Reserved

28	Reserved

30	Disconnect inactivity timer

set the length of time allowed for inactivity before the connection is hung up.

Range: 0-255 in 10 seconds

Default: 0 (disabled)

32	XON character

Set the value of XON character

Range: 0-255

Default: 17

33	XOFF character

Set the value of XOFF character

Range: 0-255

Default: 19

34	V.34 data rate (bit-rate)

Set the maximum bit rate for V.34

Range : 0-8 (2400 baud)

1-10 (3000 baud)

1-11 (3200 baud)

bit rate=((S34)+1)*2400bps

Default: 11 (28800 bps)

35	V.34 symbol rate (baud-rate)

Set the maximum baud rate for V.34

Range: 0-5

0 - 2400 baud 1 - 2743 baud (N/A)

2 - 2800 baud (N/A) 3 - 3000 baud

4 - 3200 baud 5 - 3429 baud (N/A until V.34bis)

Default: 0 (2400 baud)

36	Reserved

37	Line connection speed

0 - Attempt to connect at the highest speed

3 - Attempt to connect at 300 bps

4 - Attempt to connect at 1200 bps

6 - Attempt to connect at 2400 bps

7 - Attempt to connect at 4800 bps

8 - Attempt to connect at 7200 bps

9 - Attempt to connect at 9600 bps

10 - Attempt to connect at 12000 bps

11 - Attempt to connect at 14400 bps

12 - Attempt to connect at V.34

Default: 0

38	Delay before forced hang up

Set the delay to hang up after the disconnecting command is received.

Range: 0 - 255 seconds

Default: 20 seconds

39	Reserved

40	Reserved

41	Reserved

42	Reserved

43	Reserved

44	Reserved

45	Reserved

46	Reserved

47	Reserved

48	Reserved

82 	Reserved

86	Call failure reason code

0 - Normal disconnect; no error

4 - Loss of carrier

5 - V.42 negotiation failed to detect  an error correction modem at remote end

6 - No response to complete negotiation

9 - No common protocol

12 - Remote initiated a normal disconnect

13 - Remote modem did not respond after 10 message retransmissions

14 - Protocol violation

15 - Compression Failure

20 - Hang up by inactivity time out

91	Transmit level

Set the transmit level in -dBm

Range: 0 - 15 (-dBm)

Default: 11 (-11 dBm)

FAX Class 1 Command Set

Command	Function

+FCLASS=0	Select data mode

+FCLASS=1	Seclect facsimile Class 1 mode

+FAE?		Report active adaptive answer setting:   0 for disabled, 1 for enabled

+FAE=?		Report adaptive answer capability

+FAE=0		Disable adaptive answer

+FAE=1		Enable adaptive answer

+FCLASS=?	Report service classes supported

+FTS=n		Stop transmission and pause, 0-255 in 10 ms

+FRS=n		Wait for silence, 0-255 in 10 ms

+FTM=?	Report Class 1 transmit capabilities

+FRM=?	Report Class 1 receive capabilities

+FTH=n	Transmit data with carrier n, n = 3, 24, 48, 72, 73, 74, 96, 97, 98, 121, 122, 145, 146

+FRH=n	Receive data with carrier n, n = 3, 24, 48, 72, 73, 74, 96, 97, 98, 121, 122, 145, 146

10/08/96 gl